# CHANGE ORDER SUMMARY – OPEN COs

# FOR

# NPAC SMS FUNCTIONALITY

Rev: 224
July 01, 2026

# Preface

Originally commissioned as a working group under the NANC (North American Numbering Council) the LNPA WG (Local Number Portability Administration Working Group) dealt with Number Portability issues, processes/procedures and changes to the NPAC SMS. In December 2018 it was renamed the TOSC (Transition Oversight Sub Committee) and managed issues/changes related to the transition of NPAC from the previous vendor to iconectiv.

After the re-chartering of the NANC, the group became The Informal LNP Team until November of 2020 when the group restructured into the NPIF (Number Portability Industry Forum).  The NPIF works with the NAOWG (Number Administration Oversight Working Group) on any issues that require the involvement of NANC and continues its mission to manage processes/procedures, changes to the NPAC SMS and issues related to Number Portability.

This Change Order Summary document tracks the status of all Change Orders that were opened as part of, or after NPAC Transition (5-25-18).  Information on Change Orders Implemented/Closed prior to Transition (5-25-18) and not part of Release 3.4.8 baseline, can be found in the Change Order Summary Pre-Transition – Implemented COs document located on the numberportability.com website.  Information on Change Orders opened after Transition and Implemented/Closed after Release 3.4.8 baseline can be found in the Change Order Summary Post Transition – Implemented COs document located on the numberportability.com website

# Legend

- Release #/Target Date – Number and date of development release in which changes will be made to support Change Order

- Change Order Number – Description/Name – Number and name assigned by CMA after CO has been accepted.

- Originator – Company that created the Change Order

- Date Accepted – Date the Change Order was accepted by NPIF (Number Portability Industry Forum)

- Description – Name of the Change Order and the Business Need as defined in the Change Order itself

- Category –Category where Change Order currently resides in the process

  - Open

  - Accepted

  - Next Doc Release

  - Development Release

  - Awaiting SOW

  - Approved SOW

  - Cancel-pending

- Status – Status of Change Order shown on NPAC website

  - Closed – The Change Order was considered and rejected.

  - Open – The Change Order has been considered and there may be further discussion.

  - Requested - The LNPA TOSC has reached agreement on the Change Order and either a SOW may be requested or the requirements updates (Doc Only) will be included in a future version of the Industry Document(s).

  - Implemented – The Change Order was adopted and has been implemented in the NPAC system.   It will remain in the Change Order Summary – Open COs for 1 cycle then be moved to the Change Order Summary – Implemented COs document

- Notes – Additional detail on the Change Order status

- NPAC Level Of Effort – This field defines the Level of Effort to implement the Change Order (Low, Medium or High)

- Systems Impacted – CMIP or XML –This field indicates if there is an impact to the Local System (SOA or LSMS).  Choices are: Yes or No

- PIM # - This is the Problem Identification Management number of the PIM associated with the Change Order.

- Go To Link – This is a link to the actual Change Order Detail.

| Change Order Summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Release #/ Target Date | Change Order # - Description/Name | Category | Status | PIM # | Go To Link |
|  | CO 573 – Unauthorized LRN Creation in Non-Owned CO Codes | Accepted | Open | 162 | CO573 |
|  | CO 572 – Pooled SV Modification – 10x People/iconectiv | Approved SOW | Requested | 155 | CO572 |
| R5.4 – 1Q ‘27 | CO 570 – SPID Migration SV Counting | Approved SOW | Requested | 156 | CO570 |
| R5.4 – 1Q ‘27 | CO 567 – New Pseudo-LRN NPA-NXX-X SIC-SMURF File Email | Approved SOW | Requested | 154 | CO567 |

| # Change Order Details |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # CO # | # Originator | # Date Accepted | # Description | # Category | # Notes | # NPAC Level # Of # Effort | # Systems Impacted |  |  |  |
|  |  |  |  |  |  |  | # CMIP |  | # XML |  |
|  |  |  |  |  |  |  | # SOA | # LSMS | # SOA | # LSMS |
| CO 573 | iconectiv | 04/29/26 | # Name: # Unauthorized LRN Creation in Non-Owned CO Codes # Business Need: The CO Code Holder is the only entity authorized to establish LRNs in an NPA-NXX, unless there is a business arrangement between the CO Code Holder and another service provider. When an unauthorized service provider creates an LRN within the NPA-NXX they are misusing the LRN and are in violation of ATIS 0300065 – LRN Assignment Practices which states “The LRN shall be selected and established from a valid NPA-NXX that has been uniquely assigned to the service provider by the North American Numbering Plan Administrator (NANPA) and published in the LERG Routing Guide.” Unauthorized LRNs may result in inconsistent routing across networks, conflicting LRN to OCN mapping, erroneous call flows, default routing or translation failures, or the inability to disconnect or transfer a CO Code or Thousands-Block. See also PIM 162. # CO 573 | Accepted | - 04/29/26 NPIF Meeting - The draft Change Order for PIM 162 was reviewed by LNPA (Matt T.), accepted and assigned # 573 - If the NPIF wants to address cleanup of SVs on these LRNs, that would require a new PIM - CMA to update the CO Summary – Open COs and post the Change Order to the website - How does this CO apply to resellers? - The LRN would be created by one of the Network IDs - This CO is not code specific but is region specific 06/03/2026 NPIF Meeting - Matt T. (iconectiv) reviewed version 2 of the CO - LNPA – there are no changes currently to the SPID Migration validations - New AI – SPs and vendors to review the proposed changes to CO 573 |  | # N | # N | # N | # N |
| CO 572 | 10x People/iconectiv | 02/04/26 | # Name: # Pooled SV Modification # Business Need: Pooled TNs that are part of a Number Pool Block (NPB) cannot be modified. Any modification of routing data requires a Create-and-Activate Process. Service Providers have automated processes so that when service issues/problems occur, they move TNs to another switch. This automated process requires modifying the TNs from LRN-1 value to LRN-2 value. However, modifications are not allowed for TNs that are still associated to an NPB (i.e., the TNs contain an SV-Type of “Pool”) in the NPAC. This causes operational confusion for Service Provider personnel during a period when time-is-of-the-essence getting a customer back in service. See also PIM 155. CO 572 | Approved SOW | 02/04/26 NPIF Meeting - John N. (10X People) reviewed the reason for the change and Matt T. (LNPA) presented the draft Change Order - An SP asked what happens if a Pool Block is not in the NPAC, would this feature work? - Matt T. (LNPA) – This would work for Pooled SVs that are in the NPAC. In prior meetings we had discussed building a Create/Activate feature regardless of the status of the pool block. Consensus was reached to go forward with only this specific functionality - An SP asked what would happen if this SV is pending and also part of a SPID Migration? - The TN in question so the Modify Request of this Pooled TN would happen immediately. The SV will not be pending at all. The status would go from Sending to Active immediately - Consensus was reached to accept this CO - It was assigned #572 - CMA to add to CO Summary – Open COs and post to the website - SPs were asked to review this CO in preparation for further discussion at the March NPIF meeting 03/04/2026 NPIF Meeting - There were no questions/updates from NPIF participants regarding this CO - LNPA – This flow is very different than others and we want to ensure that it would be utilized. LNPA asked what the adoption level is for this CO? How many customers are requesting this? - John N. (10x People) – There are multiple customers that are requesting this new functionality and would utilize this feature. The CO has everything that the originator is requesting - There were no objections to change this CO to Requested status - CMA to update CO Summary – Open COs and change status on website - New AI – NPIF co-chairs to ask NAPM LLC to request an SOW from the vendor for CO 572 |  | # N | # N | # Opt | # N |
| CO 570 | iconectiv | 08/06/25 | # Name: # SPID Migration SV Counting # Business Need: Although the SPID Migration Quota – All regions tunable was increased from 25 to 49 migrations/week in March 2025, Service Providers are scheduling more SPID Migrations than desired across available SPID migration windows to stay under the current maximum SV record count of 500,000 per region, as defined by the regional SPID Migration Quota – SVs tunable. This spreading out of SPID Migrations caused by the current SPID Migration SV quota causes additional workload on teams to manage and coordinate migration activities, including the HD, the SP’s System Engineers, TN/Code Admins, PMs, LNP PortIn/PortOut teams, and the SP’s network providers. The SPID Migration Quota – SVs tunable currently defines the maximum number of SVs within a region for a given SPID Migration maintenance window, but the SV/NPB records that may be migrated by the NPAC SMS can differ from the records migrated by a local system. Local systems manage SV records for non-pool SVs (LNP Type of LSPP and LISP) and also manage NPB records. Since pool SVs (LNP Type of Pool) are not sent to or maintained by local systems, including pool SVs in SPID migration quotas can limit the overall number of local system records that can be updated in weekly SPID migrations. In contrast, the NPAC SMS does maintain pool SV records in addition to the non-pool SV and NPB records managed by local systems, so the quantity of pool SVs that need to be updated during a SPID migration continues to be a consideration for the LNPA. Because the number of records migrated by local systems and NPAC SMS can differ significantly, and because the SPID Migration calendar fills up as SPs spread out SPID Migrations to stay under the current SV quota, the industry could benefit by using different SPID Migration quotas to represent an NPAC SMS maximum and a local system maximum. See also PIM 156. CO 570 | Approved SOW | 08/06/2025 NPIF Meeting - Draft CO (PIM 156 follow up) was reviewed by LNPA (Matt T.), accepted and assigned # 570 - This Change Order recommends creating a new SPID Migration tunable (Local Systems SV Quota Tunable parameter) - New AI – LNPA to investigate what SVs are included in the post-migration report - CMA to add # to CO, update CO Summary documents and post to website 09/10/2025 NPIF Meeting - 08062025-06 – LNPA to investigate what SVs are included in the post-migration report - Matt T. (iconectiv) reviewed the updates to CO 570 - Consensus was reached on proposed updates to CO 570 - Consensus was reached to move this CO to requested status - This AI is now closed - New AI – NPIF co-chairs to reach out to the NAPM LLC to ask them to request an SOW for CO 570 from the vendor 10/08/2025 NPIF Meeting - 09102025-04 – NPIF co-chairs to reach out to the NAPM LLC to ask them to request an SOW for CO 570 from the vendor - An SOW was requested |  | # N | # N | # N | # N |
| CO 567 | iconectiv | 10/02/24 | # Name: New Pseudo-LRN NPA-NXX-X SIC-SMURF File Email Business Need: Currently, though NPBs having a pseudo-LRN (LRN = 000-000-0000) and the associated NPA-NXX-Xs are migrated when the NPA-NXX of the NPB exists in the NPA-NXX Selection Input Criteria SPID Migration Update Request Files (SIC-SMURF file), some local systems are only migrating the NPA-NXX-Xs that exist in the NPA-NXX-X SIC-SMURF file, which would not be the case for pseudo-LRN NPBs. For thousands-blocks, in addition to migrating a pseudo-LRN NPB object based on its NPA-NXX being included in the NPA-NXX SIC-SMURF file, the associated NPA-NXX-X object is migrated as well, since the two objects must be kept in sync. Additionally, there may be occasions an NPA-NXX-X that indicates it is a pseudo-LRN block is migrated by the NPAC, but the NPB does not exist. In these cases, the local system does not have a way to determine that the -X is related to a pseudo-LRN NPB, and should be migrated in the SOA/LSMS, because the -X download sent to the SOAs/LSMSs does not include a pseudo-LRN indicator. Because some local systems may not migrate -X objects for a pseudo-LRN block, consensus was reached during the discussion of PIM 154 that a new pseudo-LRN NPA-NXX-X SIC-SMURF file is needed. See also PIM 154. CO 567 | Approved SOW | 10/02/2024 NPIF Meeting # Matt T. (iconectiv) reviewed the draft CO # The Description of Change defined in the CO is that a 4th SIC-SMURF file would be created and put in the SFTP directory for all SPs # Draft CO was discussed, accepted and assigned # 567 # CMA to update CO Summary – Open COs and post a copy of the CO to the website # 09/10/2025 NPIF Meeting # John N. (10X People) asked if this CO can be moved to Requested status # Consensus was reached to move CO 567 to requested status # New AI – NPIF co-chairs to reach out to the NAPM LLC to ask them to request an SOW for CO 567 from the vendor # 10/08/2025 NPIF Meeting # 09102025-03– NPIF co-chairs to reach out to the NAPM LLC to ask them to request an SOW for CO 567 from the vendor # An SOW was requested |  | # OPT | # OPT | # OPT | # OPT |